ANNEXURE – 3

ANALYSIS REPORT FORM

SPECIFICATIONS FOR PRILLED / GRANULAR UREA (BULK) – FERTILISER GRADE

1. Description

The material shall be in the form of white, free-flowing granules, prills, pellets, or shots, and shall be:

- Free from visible impurities and dust

- Chemically treated and coated to reduce hygroscopicity

2. Free-Flowing Criteria (at the Time of Unloading)

The material shall be considered free-flowing if:

- Prills/granules are separate from each other

- Lumps or crusts formed in vessel hatches can be broken into separate prills/granules by slight finger pressure

- Cliffs formed near hatch walls can be broken by slight pressure using bamboo sticks or iron rods

3. Product Form & Shipment Conditions

- Urea supplied shall be either prilled or granular

- Cargo-wise mixing is not permitted

- The entire vessel must contain the same type of urea

- Combined shipment of prilled and granular urea, even in separate holds, is strictly prohibited

⚠️ Note:
Any deviation from the above conditions must be immediately report to IPL and clearly mentioned in the analysis report.

4. Technical Specifications & Test Results

| S. No. | Characteristics | Requirements | Result |
| --- | --- | --- | --- |
| 1 | Moisture (% by weight), maximum | 0.50 % |  |
| 2 | Total Nitrogen (% by weight, dry basis), minimum | 46.00 % |  |
| 3 | Biuret (% by weight), maximum | 1.50 % |  |
| 4 | Particle Size – Prilled | See details below |  |
|  | Particle Size – Granular | See details below |  |

5. Particle Size Requirements

A. Prilled Urea

- ≥ 90% by weight shall:

  - Pass through 2.8 mm IS Sieve (Tyler Sieve No. 7)

  - Be retained on 1.0 mm IS Sieve (Tyler Sieve No. 16)

- ≤ 5% by weight shall pass through 1.0 mm IS Sieve

B. Granular Urea

- ≥ 90% by weight shall:

  - Pass through 4.0 mm IS Sieve (Tyler Sieve No. 5)

  - Be retained on 2.0 mm IS Sieve (Tyler Sieve No. 9)

- ≤ 5% by weight shall be below 2.0 mm IS Sieve

6. Methods of Analysis

| S. No. | Parameter | Method of Analysis |
| --- | --- | --- |
| 1 | Nitrogen | AOAC (1995), Method 2.4.03 |
| 2 | Biuret | AOAC (1995), Method 2.4.23 |
| 3 | Moisture | D.4, TFI (1982) |
| 4 | Particle Size | IV A, TFI (1982) |
| 5 | Sampling | AOAC (1995) 2.1.01 OR IC-1 / IC-2, TFI (1982) OR Schedule II-A, FCO (1985) |